| ENGINEERING HAND BOOK |  |  | : |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| REVISION |  |  | : 0 |  |  |  |
| Material Property Database in MS-Access with translator to I-DEAS material DB |  |  |  |  |  |  |
| TYPE |  |  | : DOCUMENTATION |  |  |  |
| ACCESS |  |  | : PUBLIC |  |  |  |
| 07-06-05 | 0 | FIRST ISSUE |  | SB | NM | NRR |
| DATE | REVISION | REVISION DETAILS |  | PREPARED BY | CHECKED BY | APPROVED BY |

1. Introduction

The present documentation describes the structure of a database created with MS Access containing some commonly used material properties of ASME materials frequently used for the purpose of Finite Element Analysis. A few Visual Basic codes have also been written in order to read and interpolate the data and to convert the database to I-DEAS universal file format. The database can be edited, modified and appended with as many number of new materials or property sets without necessity of making much change to the VB codes. However in order to incorporate new properties into the database some changes are required in the codes for their proper functioning. A brief discussion on these follows.

2. Structure of the Database

The database makes use of one main table, ‘Materials’, containing the material names, compositions and references to parts of the ‘Data_’ tables containing the property values. The ‘Data_’ tables may be viewed as pools (or collections) of several sets of property values, to which references are made in the ‘Materials’ table.

There is also a set of ‘ID_’ tables, whose soul purpose is to create one-to-many relationships between the ‘Materials’ and ‘Data_’ tables, which would otherwise had been many-to-many relationships. The user of the database may not be concerned much about the ‘ID_’ tables.

Below are the salient features of the structure of the database.

[ Notes A:

- A ‘Record’ refers to an entry in a table.

- The term ‘Field n’ refers to the column (n+1) of the particular table or record.

- The term ‘property data set’ refers to a set of property values at different points of temperature. Each material property ‘Data_’ table contains several such sets, which in turn are referenced by the materials in the ‘Materials’ table to describe the corresponding property. For a particular property, a particular set has an unique ID number.]

- The Field 0 (‘Material Name’) of the ‘Materials’ table contains the names of the different materials, and Field 6 (‘Composition’) contains the composition.

- For each record in the ‘Materials’ table, the Fields 1, 2, 3, 4 and 5 contains a reference number or ‘ID’ of the properties ‘Modulus of Elasticity’, ‘Thermal Conductivity’, ‘Coefficient of Thermal Expansion’, ‘Thermal Diffusivity’ and ‘Allowable Stress’ respectively. The ID refers to a particular set of values in the respective ‘Data_’ table which are the property values of the material.

- There are 5 ‘Data_’ tables which contain several sets of the property values. Each set (identified by an ID) contains the property values at different temperature points.

- All the units used in the database are same as those in ASME code in Section II Part D. Temperatures are in Fahrenheit.

- A particular data set may be referenced by more than one material. Making changes in that data set will hence cause change in all those materials’ that property.

fig – 1: Structure of the database – Displaying a property data set –

Modulus of Elasticity values for material SA 182 Gr F11

3. Working with the database Manually

3.1. Retrieving a Value from the database

To extract a value from the database proceed through the following steps:

- Open the ‘Materials’ database and find out the material name of which the property value is sought.

- Note the ID number that is present in a property field, the value of which is being sought.

- Open the ‘Data_’ table corresponding to that property.

- Scroll down and search for the set of records for which the ‘ID’ fields contain the same ID number as that noted from the ‘Materials’ table in step ii) (i.e. the property data set). All such records’ Field 1 (temperature) and Field 2 (property value) gives the property values at different temperature points.

3.2. Editing the already existing Values

To edit a property value of the material, or the temperature entries, just go to the corresponding position as described in 3.1. and edit as required.

3.3. Adding values to an already existing material’s property

To add a temperature and value point for an already existing material in the database, do the following:

- Open the ‘Materials’ database and find out the material name of which the property value is sought.

- Note the ID number that is present in a property field, the value of which is being sought.

- Open the ‘Data_’ table corresponding to that property.

- Add a record at the end of the ‘Data_’ table with Field 0 equal to the ID number noted in step ii) and Field 1 and Field 2 equal to the temperature and property value to be added. Note that you must use F as temperature units and the unit system in accordance to ASME Sec II Part D for the property values.

- Sort the database first with respect to Temperature, next with respect to ID.

3.4. Adding a new material to the database

To add a new material to the database, proceed as follows:

- Open the ‘Materials’ database and add a new record at the end of the database with Field 0 (Material Name) as the name of the material.

- For a particular property, check whether the property data set intended to be inputted already exists in the corresponding ‘Data_’ property table. [There is no definite procedure for doing this. One may check it from knowledge of the material groupings made in the ASME code, or may go through all the property data sets in the ‘Data_’ table and find out if the dataset already exists]

- Hence, two cases may arise now:

Case I: The property data set for the material’s particular property already exists:

In such a case, just put the ID number of the property data set into the appropriate field beside the material’s name in the ‘Materials’ table.

Case II: A new property data set needs to be added for the particular property:

Pursue the following steps:

- Open the ‘Data_’ property table.

- Scroll down the records to find what is the maximum number that has been used as ID number in the table. Hence determine the next available ID.

- Open the corresponding ‘ID_’ property table and make sure that the ID number exists. If not, add it.

- Open the  ‘Materials’ table and put the foresaid ID number in the appropriate field beside the material.

- Go back to the ‘Data_’ table and keep adding records with the same ID number in the ‘ID’ field, and temperature & property values in the fields 1 & 2 respectively.

4. Working with the forms

4.1. Converting the MS Access database to I-DEAS Universal file of material properties

The database can be converted to I-DEAS 11NX universal file. The steps involved are:

- The files ‘IDEAS_material_format.dat’ and ‘IDEAS_materials_header.dat’ should present in the same folder as the access database file ‘Material Properties.mdb’. If they are not present, copy them into the folder of the database.

- Double-click to open the ‘Export database to IDEAS UNV file’ form. Select the path and file name for exporting the database using the ‘Browse…’ button.

- Click ‘Export database’ to export database.

4.2. Finding and Interpolating values using different units

For seeking a particular property value at a  particular temperature by linear interpolation upon the database values, perform the followings:

- Open the ‘Seek/Interpolate’ form.

- Select the material name and the property name from the respective drop-down lists.

- Select the desired units for temperature and the property value.

- Click the ‘Seek’ button.

Notes B:

  - All the values displayed in the form are in the units selected by the user.

  - The values displayed do not get updated unless the ‘Seek’ button is clicked.

  - The user may wish to open the ‘Materials’ table or the property ‘Data_’ table directly at the positions from where the last values have been sought, by clicking the ‘Open 'Materials' Table’ and ‘Open Property Value database Table’ buttons respectively.

4.3. Listing, adding, editing and deleting records of the existing database

The ‘List/Edit/Add’ for gives the user the ultimate authority on the database in an user-friendly manner. Though care has been taken to ensure that the form works properly under all circumstances and don’t crash, it is always advisable to create a backup of the database before editing/modifying it with this form.

To add, edit and/or delete records from a property data set using this form, pursue the following steps:

- Open the ‘List/Edit/Add’ form.

- Select a material from the drop-down list of which a property data set is intended to be edited/modified.

- Select the material property from the drop-down list.

- Now note:

  - This has loaded the temperature & value points of the property data set assigned to the selected material’s selected property, which will hence be displayed in the list.

  - In the upper half of the right side of the list, the user may see the ID of the currently loaded property data set.

    - Click on a record in the list. This will transfer it’s value to the ‘Temperature’ and ‘Property Value’ text boxes on the right side of the list.

    - Make change to the values in the text box or type completely new values and click the ‘Add Record’ or ‘Update Record’ Button. Click ‘Delete Record’ to delete the selected record.

    - After the desired changes have been made, click the ‘Update database with current list’ button at the bottom of the form.

    - A dialog box will pop up asking for confirmation whether to overwrite/update. Click ‘yes’ to save the changes.

Notes C:

      - ‘Add Record’ will add a new temperature and property value point. The record will be added at the position of currently selected record in the list. However the position of a record is of not much importance other than ease of viewing. ‘Update Record’ will update the presently selected record with the values in the text box. ‘Delete Record’ will delete the presently selected record.

      - The user may wish to view/enter values in different unit systems by choosing the appropriate units from the ‘Material Property unit’ and ‘Temperature unit’ drop-down lists in the upper right side of the form.

      - The user may chose to work in any unit system throughout without worrying what the unit system of the actual database is. The program takes total care of all the unit conversions.

      - The user may click the ‘Sort List’ button to view the list sorted in ascending order of temperature.

      - The user must click the ‘Update database with current list’ button to save the changes made to the property data set, before he/she can switch to some other material or property. Once switched to other material or property, the unsaved changes made to the already load set will be lost.

      - Though, by default the ID number of the data set to which the currently loaded list to be saved will remain the original ID of the Set, the user may opt to save the list to a data set with a different ID by typing the desired ID number in the ‘Save this property with ID set number’ text box. The ID number should be a positive integer less than 1000. Type ‘NEW’ or ‘0’ to create a new data set with the lowest unoccupied ID number.

4.4. Adding a new material to the database and creating, editing it’s property values

It is also possible to add a new material to the database using the ‘List/Edit/Add’ form. To do so, the steps involved are:

- In the ‘List/Edit/Add’ form type the name of the new material to be added in the ‘Material Name’ box.

- Type the composition of the material in the ‘Composition’ box. Or select an already existing composition from the drop-down list. [For typing new material composition, use the standard format as used for the other materials.]

- Click the ‘Add Material’ button to add the material to the database.

- Do not change the material name or close the form unless the following points have been considered. Please read on …

Depending on the composition of the material the program may assign some default values to the ID numbers for the different properties of the material if the composition already existed in the database. The user may however opt to change the property set ID numbers for the material to some other value, or may create a new property data set by entering new values. These are to be done in the following steps:

- Select the property whose ID/values is to be changed/modified.

Note: If some default ID number had been assigned to the property by the program, the corresponding data set will be displayed in the list and the ID number will be displayed beside the list. Else the list will remain blank and the ID number will be displayed as 0.

- If required, to load an already existing property data set to the list, type the ID number of the set into the ‘Load an already existing property Set’ text box and click ‘Load set to list’ button. This will load the set to the list.

- Now the user may edit the list as described in steps v) and vi) of 4.3. New temperature and value points may be added as desired.

Tips: To Add several temperature and value points to the list, the following key combinations may be useful:

- Use the ‘Space bar’, ‘up arrow’ or ‘down arrow’ keys to toggle between the ‘Temperature’ and ‘Property Value’ text boxes.

- Use the Enter key (or Carriage Return) to add the presently typed temperature and property value to the list.

- Once the list contains the desired values, click the ‘Update database with current list’ button to add the list to the database as a property data set.

Note: The ‘Save this property data set with ID number’ text box indicates the ID of the data set to which the values will be saved. The user may chose to use a different number or may type ‘NEW’ or ‘0’ to create a new data set with the lowest unoccupied ID number. However, if unsure, the user may leave this value as it is.

- A data set addition/modification confirmation message box will pop-up. Click ‘Yes’ to add/modify.

Notes: All the notes mentioned under ‘Notes C’ will also apply here.

6. Appendix

6.1. User Accounts for the database

There are two user accounts for the database, namely ‘User’ and ‘Admin’.

The ‘User’ account does not have any password. But the database and forms can only be read/accessed from the account, and cannot be edited or modified. Thus, for the form ‘List/Edit/Add’, only the Listing feature will work properly. The other two forms will work as desired.

The ‘Admin’ account has full access to the database and all the forms & tables can be modified/edited from this account. This account is however password-protected.

6.2. Adding a new material property to the database

To add a property other than the 5 provided ones, the user need to create a new column in the ‘Materials’ table and a new ‘Data_’ and an ‘ID_’ table for that property. However in such case the forms provided with the database will no more work as desired. To make them work correctly the user may need to study their codes and make appropriate changes in them. The codes contain self-explanatory variable names and comments at suitable places so that their interpretations become easier.

6.3. Changing the names of the fields in the table

It is advised not to make changes to the names of the different fields of the tables. However, in case this is absolutely necessary, corresponding appropriate changes should be made in the codes of the forms wherever the names of the fields appear in order to make the forms work properly.

6.4. Restrictions of the ‘List/Edit/Add’ form

The ‘List/Edit/Add’ form has some restrictions. They are primarily to prevent accidental loss/damage of the database. The restrictions are:

- The form cannot be used to delete a material from the database.

- The form cannot be used to delete a data set. However it can be used to modify it’s records, add to the records or delete records one by one.

- The form cannot be used to modify the name, composition or the IDs assigned to the properties of an already existing material. Any of these parameters/values are to be assigned at the time the material has been created.

6.5. Restrictions on the database for the code to work properly (Limitations)

The following constrains should be maintained on the database to ensure that the forms work properly.

- The ID number of the property data sets must be a positive integer less than 1000.

- There shouldn’t be more than 100 records in each data set.

- The standard format should be used for representation of material composition.

If the above restrictions are to be relaxed, the corresponding changes must be made to the codes of the form.